DESIGN JOURNEY

v5 Systolic Array Controller

FSM-based operand scheduler for a 2×2 systolic array — from a broken first assumption to a verified, timing-correct controller.

| REVISION v5 | ARRAY SIZE 2 × 2 | STATUS Verified |
| --- | --- | --- |

# 1. Objective

Build an FSM-based controller that feeds a 2×2 systolic array with the correct operand schedule for matrix multiplication — driving the PE grid's inputs, tracking completion, and flushing the pipeline cleanly.

# 2. The Journey

This design went through two real attempts. The first one looked reasonable, passed a basic sanity check, and was wrong for a reason that only became clear under a waveform viewer. The second one is what's running today.

## Attempt 1 — The Timing-First Assumption

The initial design used the sequence below, on the assumption that the bug — whatever it turned out to be — was a timing problem: not enough cycles for data to propagate through the array.

| IDLE | CLEAR | STREAM0 | WAIT0 | STREAM1 | WAIT1 | DONE |
| --- | --- | --- | --- | --- | --- | --- |

Why it was wrong The bug was not insufficient waiting time — it was incorrect operand scheduling. PE01 and PE10 need operands arriving from different directions in the same cycle. No amount of extra WAIT states can fix operands that are misaligned to begin with.

## The Turning Point

Adding WAIT states treated the symptom, not the cause. Systolic arrays are timing-driven architectures in a stricter sense than “wait longer” implies — every PE needs the right operand on the right cycle, from the right direction, or the array computes garbage regardless of how long it's given to settle. That reframing is what led to rebuilding the schedule itself instead of the wait logic around it.

## Attempt 2 — Fixing the Schedule, Not the Delay

The corrected design replaces the WAIT/STREAM alternation with an explicit, cycle-accurate stream of operand pairs. Each cycle below is (a-row-input, a-col-input, b-row-input, b-col-input) feeding the array's edges:

| Cycle | PE(row,col) inputs — a / b |
| --- | --- |
| 0 | (a00, 0, b00, 0) |
| 1 | (a01, a10, b10, b01) |
| 2 | (0, a11, 0, b11) |
| 3 | (0, 0, 0, 0) |

The final FSM reflects this directly — one state per cycle of the schedule, rather than generic stream/wait pairs:

| IDLE | CLEAR | STREAM0 | STREAM1 | STREAM2 | STREAM3 | WAIT | DONE |
| --- | --- | --- | --- | --- | --- | --- | --- |

# 3. Controller Responsibilities

- Clear accumulators before a new matrix multiply begins

- Generate the cycle-accurate operand stream shown above

- Drive enable, busy, and done handshake signals

- Flush the pipeline fully before signaling completion

# 4. Verification

Once the schedule was fixed, verification focused on proving it held across the cases that would expose alignment errors, not just “does it produce roughly the right numbers.”

| Test case | Purpose |
| --- | --- |
| Positive matrix multiplication | Baseline correctness |
| Zero matrix | Confirms clean accumulator clear |
| Identity matrix | Isolates pass-through operand routing |
| Signed arithmetic | Checks sign handling through the PEs |
| Boundary values (+127 / −127) | Checks saturation / overflow behavior at range edges |

- Used GTKWave to step through operand propagation cycle-by-cycle and confirm alignment

- Deliberately separated controller bugs from datapath bugs before debugging either

- Built a self-checking testbench using tasks, verified against all cases above

# 5. Mistakes and Learnings

What I'd tell myself at the start “It's not working” is not the same as “it needs more time.” Adding WAIT states was the fastest lever to pull, but it was solving the wrong problem — the real fix required re-deriving the schedule. Systolic arrays are timing-driven architectures: correctness lives in per-cycle operand alignment, not in total elapsed cycles. Separating controller bugs from datapath bugs early made root-causing far faster once I stopped assuming it was one or the other. A self-checking testbench across varied cases (zero, identity, signed, boundary) caught issues a single “happy path” test would have missed.

# 6. Future Work

- Parameterize array size to N×N instead of fixed 2×2

- Add a memory / AXI interface for streaming operands from external memory

- Support continuous streaming (back-to-back matrix multiplies without a full drain)

- Add configurable data width